- Signia Powered Stapler: https://www.youtube.com/watch?v=pBKIm3YjY8c

  - Key thing for staff: Before unloading make sure Jaws of reload are open and straight.

- TA Stapler: https://www.youtube.com/watch?v=ikbG1ADVhI4

  - Key: The first squeeze is simply grasping the tissue and the second squeeze of handle will fire the staples. TA = Twice Always.

- Tri-Staple EEA: https://www.medtronic.com/content/dam/covidien/library/us/en/product/surgical-stapling/eea-stapler-tri-staple-technology-inservice-guide.pdf

  - Key 1: Make sure orange bar is covered on approximation of the tissue.

  - Key 2: When releasing tissue/anvil – turn left 4 half turns until you feel a tactile click which indicates the anvil top is tilted.

  - Key 3: After the anvil is tilted and you are ready to remove the device – pull straight back. If you feel tension – push the device forward an inch and then remove straight back.

https://www.youtube.com/watch?v=Kv8UPQj8zN4