Setup notes

Sunday, February 7, 2016

1:44 PM

Network setup

eth1: connect this to the network. DHCP

•        New Ulm's IP: 192.168.0.25

eth0: static, this is the main interface for the camera network

•        IP address: 10.60.10.1

eth0:1 static, this is the interface used to configure cameras from default settings to our                                 destination camera network

•        IP address: 192.168.1.1

wlan0: onboard wifi.

•        IP address: 10.60.10.2

Firewall

•        Rules defined in: /etc/iptables.rules

•        Rules are loaded prior to any interfaces being brough up during boot

◦        /etc/network/interfaces

▪        pre-up iptables-restore < /etc/iptables.rules

◦        These rules prevent eth1, eth1:1 and wlan0 from communicating with eth0

User setup

Users:

•        alec

•        karl

Both users have been added to the sudoers group

Drive setup

OS Drive

•        120GB SSD

•        Partitions

◦        1, mounts to /boot, ext2 formatted, 1GB size. Boot partition

◦        2, swap, swap formatted, 8GB size

◦        5, mounts to /, ext4 formatted, 40GB size. Root partition

◦        6, no mount point, not formattted, 40GB size

▪        To be used as a clone of 5

◦        7, no mount point, not formatted, 31GB size

▪        To be used to store clonezilla image

•        Clonezilla image will dump an image partition 5 to partition 6 and then reboot into partition 5

•        UUIDs

◦        1: 6a8941dd-b0db-4a23-a71e-01a9af881e43

◦        2: 846b6246-9cfa-463a-9374-979ef366836c

◦        5: c6a5af59-43d4-4b9e-ae97-571cd757d09f

•        /dev/disk/by-id:

◦        ata-KINGSTON_SV300S37A120G_50026B775C027972

◦        wwn-0x50026b775c027972

Storage Drives

NOTE: Storage array is NOT currently set up, as an additional Molex->SATA power converter is needed

•        2x 5TB WD Black drives

◦        RAID1 using MDADM

▪        MDADM emails alerts to:

•        alec@papierniak.net

•        karl@papierniak.com

◦        S.M.A.R.T. alerts enabled

▪        Alerts emails to

•        alec@papierniak.net

•        karl@papierniak.com

OS Setup

Software installed

•        vim

•        tmux

•        iftop

•        iotop

•        htop

•        rsync

•        sudo

•        virtualbox

To add an additional network on the camera-only network

•        vim /etc/network/interfaces

•        add the following lines to the end of the file, replacing X with the next number

◦        i.e. if eth0:4 is the last entry in the file, replace X with 5, resulting in eth0:5

◦        auto eth0:X

allow-hotplug eth0:x

iface eth0:X inet static

address 10.100.100.1

netmask 255.255.255.0

broadcast 10.100.100.255

◦        In the above example, replace '10.100.100.1' with the desired network configuration

•        Bring the new interface up:

◦        (as root): ifup eth0:X

•        On reboot, the new interface will automatically be added to the startup

•        Add an entry to: /etc/iptables.rules to prevent the new interface from reaching the normal network

◦        Normal network lives on eth1

Setup of WiFi network with wlan0

•        # apt-get install hostapd